Pipeline Setup Checklist

Every value to obtain and every check to pass before the robot-inspection pipeline will run. Tick each box; write your value in the blank column.

# 1.  Values to obtain (write them down)

Credentials & access:

| Item | Where to get it / example | Your value |
| --- | --- | --- |
| Spot IP address | From your Spot admin / on Spot's GXP: 192.168.50.3 |  |
| Spot username | Boston Dynamics account for the robot |  |
| Spot password | Boston Dynamics account for the robot |  |
| GitHub repo access | woopers6/robot-inspection (login/token if private) |  |
| Trimble Perspective license | Installed on the laptop, X7 paired |  |

Network values (see the network setup guide):

| Item | Where to get it / example | Your value |
| --- | --- | --- |
| Beryl Wi-Fi SSID | You choose, e.g. spot-field (5 GHz) |  |
| Beryl Wi-Fi password | You choose (WPA2/WPA3) |  |
| Beryl LAN subnet / gateway | 10.0.0.0/24 gateway 10.0.0.1 |  |
| Jetson IP (on Beryl) | Static lease 10.0.0.10 |  |
| Laptop dongle IP (on Beryl) | Static lease 10.0.0.20 |  |
| Spot payload (GXP) IP | 192.168.50.3 (confirm your payload config) |  |
| X7 Wi-Fi SSID / password | The scanner's own AP (for Perspective) |  |
| ROS_DOMAIN_ID | 42 (Jetson only) |  |

# 2.  config/field.env values to set / confirm

On the Jetson, in config/field.env. The localization block is the critical one for camera-only operation.

| Item | Where to get it / example | Your value |
| --- | --- | --- |
| SPOT_IP | Spot's address (192.168.50.3) |  |
| SPOT_USERNAME / SPOT_PASSWORD | Spot login (blank = fill in) |  |
| TRIMBLE_WINDOWS_URL | http://10.0.0.20:8765 (laptop dongle IP) |  |
| ROS_DOMAIN_ID | 42 |  |
| IMAGE_TOPIC | Must match your camera driver (ros2 topic list) |  |
| MAP_DEPTH_POINTS_TOPIC | The camera's point cloud, e.g. /depth/points |  |
| ROBOT_GOAL_BRIDGE | false for supervised (recommended first) |  |

Localization = camera only (do NOT change these):

| Item | Where to get it / example | Your value |
| --- | --- | --- |
| SPOT_LOCALIZATION | false (Spot's leg-encoder odometry OFF) |  |
| DEPTH_LOCALIZATION | true (Oak-D VIO is the localizer) |  |
| ROBOT_WORLD_FRAME | depth_odom |  |
| DEPTH_ODOM_TOPIC | /oak/odom (must match your camera's VIO topic) |  |
| FUSED_LOCALIZATION | false (on would re-add Spot's pose) |  |

CRITICAL: localization now comes only from the Oak-D. Your camera launch MUST publish visual-inertial odometry on /oak/odom (on-device VIO or an RTAB-Map/VIO node). No /oak/odom = no localization = no map.

# 3.  Jetson - install & verify

| ✓ | What to check | How |
| --- | --- | --- |
| ☐ | Ubuntu 24.04 / JetPack 6 | cat /etc/nv_tegra_release |
| ☐ | Repo cloned to ~/ros2_ws | ls ~/ros2_ws (src scripts config docs tools) |
| ☐ | ROS 2 Jazzy installed | ros2 --help |
| ☐ | Python libs | pip3 show numpy opencv-python pyyaml |
| ☐ | Spot SDK + scan libs | python3 -c 'import bosdyn.client, laspy' |
| ☐ | Depth-camera driver (depthai-ros) | ros2 pkg prefix depthai_ros_driver |
| ☐ | Workspace builds | cd ~/ros2_ws && colcon build --symlink-install |
| ☐ | Camera point cloud is live | ros2 topic hz /depth/points |
| ☐ | Camera VIO odometry is live | ros2 topic hz /oak/odom (localization!) |
| ☐ | Topic names match field.env | ros2 topic list ¦ grep -E 'rgb¦depth¦points¦odom' |
| ☐ | No-hardware sim runs | ./scripts/run_field.sh mission --sim synthetic |

# 4.  Windows laptop - install & verify

| ✓ | What to check | How |
| --- | --- | --- |
| ☐ | Trimble Perspective scans the X7 on its own | Take a test scan first |
| ☐ | Python 3.12 installed (Added to PATH) | py --version |
| ☐ | Repo downloaded (ZIP or git clone) | robot-inspection folder present |
| ☐ | Bridge deps installed | Run 'Install Windows Dependencies.bat' |
| ☐ | Bridge app launches | py windows_app.py -> dashboard on :8765 |
| ☐ | Bridge settings filled | jetson_host 10.0.0.10, windows_advertise_host 10.0.0.20 |
| ☐ | export_dir points at Perspective's export folder | Settings tab |
| ☐ | auto_transfer is OFF | Settings tab |
| ☐ | Firewall allows inbound 8765 | New-NetFirewallRule ... -LocalPort 8765 |
| ☐ | USB Wi-Fi adapter on the Beryl | Built-in Wi-Fi stays on the X7 |

# 5.  Hardware & wiring

| ✓ | What to check | How |
| --- | --- | --- |
| ☐ | Jetson Orin Nano has Wi-Fi (M.2 AX210) | nmcli device status -> wlan0 |
| ☐ | Jetson Ethernet wired to Spot GXP | ping 192.168.50.3 from Jetson |
| ☐ | Oak-D Pro connected + udev rule set | USB 3; 80-movidius.rules |
| ☐ | Camera->body transform (extrinsic) published | ros2 run tf2_tools view_frames |
| ☐ | Beryl AX powered, LAN 10.0.0.0/24, AP isolation OFF | admin at 10.0.0.1 |
| ☐ | Static leases set (Jetson .10, laptop .20) | Beryl Address Reservation |
| ☐ | Spot powered, tablet + E-Stop in hand |  |
| ☐ | Trimble X7 + tripod ready, paired in Perspective |  |

# 6.  Network connectivity - test both directions

| ✓ | What to check | How |
| --- | --- | --- |
| ☐ | Jetson reaches the laptop bridge | curl http://10.0.0.20:8765/status |
| ☐ | Jetson reaches Spot | ping 192.168.50.3 |
| ☐ | Laptop can SSH the Jetson | ssh <user>@10.0.0.10 "echo ok" |
| ☐ | Perspective still sees the X7 | On the laptop's X7 Wi-Fi |
| ☐ | Preflight passes | ./scripts/field_preflight.sh mission |

# 7.  First run - supervised (no autonomous driving)

| ✓ | What to check | How |
| --- | --- | --- |
| ☐ | Perspective + bridge app running on the laptop |  |
| ☐ | Mission starts with motion OFF | ROBOT_GOAL_BRIDGE=false ./scripts/run_field.sh mission |
| ☐ | Map grows as you walk Spot by tablet | ros2 topic echo /mission/state |
| ☐ | Robot parks at a scan spot and WAITS | expected - operator paced |
| ☐ | 'Next scan location' releases it to the next | press in the bridge app after scanning |
| ☐ | Returns to start -> AWAITING_UPLOAD | ros2 topic echo /mission/state |
| ☐ | Upload E57 + Finish closes the mission | bridge app button |

Autonomous driving (ROBOT_GOAL_BRIDGE=true + spot_sdk) is NOT yet wired for camera-only localization: goals are in depth_odom but Spot commands in its own frame, with no TF linking them. Run supervised until that motion bridge is added.

# 8.  Optional - tuning values to decide

| Item | Where to get it / example | Your value |
| --- | --- | --- |
| EXPLORATION_RADIUS_M | How far to map (default 40) |  |
| MISSION_MAX_EXCURSION_M | >= exploration radius (default 40) |  |
| SCAN_MAX_STATIONS | How many scans to take (default 3) |  |
| SCAN_MIN_SEPARATION_M | Spacing between scans (default 8) |  |
| SCAN_MIN_OPENNESS / SCAN_OPENNESS_RADIUS_M | Vantage clearance (0.6 / 3.0) |  |
| SCAN_CENTRALITY_SCALE_M | Bias toward structure centre (default 10) |  |
| PLANNER_TRAVEL_DECAY_M | Greedy vs. reach (default 8) |  |